# FSM State Diagram

## Purpose

This document defines the Finite State Machine (FSM) used by the traffic light controller. The FSM determines the traffic light outputs and the conditions for moving from one state to another.

## State Definitions

| State | Description |
| --- | --- |
| S0 | North–South Green, East–West Red |
| S1 | North–South Yellow, East–West Red |
| S2 | North–South Red, East–West Green |
| S3 | North–South Red, East–West Yellow |
| S4 | Emergency: North–South Green, East–West Red |
| S5 | Emergency: North–South Red, East–West Green |

## FSM State Diagram

FSM Operation

After reset, the controller enters S0.

The timer controls normal state changes.

If emergency_ns becomes HIGH, the FSM immediately enters S4.

If emergency_ew becomes HIGH, the FSM immediately enters S5.

The FSM remains in an emergency state while the corresponding emergency signal stays HIGH.

When the emergency signal goes LOW, the FSM returns to S0 and resumes the normal sequence.