# Output Table

## Purpose

The output table defines the status of every traffic light in each FSM state. A logic '1' means the light is ON and a logic '0' means the light is OFF.

## Output Signal Definitions

| Signal | Description |
| --- | --- |
| ns_red | North–South Red Light |
| ns_yellow | North–South Yellow Light |
| ns_green | North–South Green Light |
| ew_red | East–West Red Light |
| ew_yellow | East–West Yellow Light |
| ew_green | East–West Green Light |

## FSM Output Table

| State | NS Red | NS Yellow | NS Green | EW Red | EW Yellow | EW Green |
| --- | --- | --- | --- | --- | --- | --- |
| S0 | 0 | 0 | 1 | 1 | 0 | 0 |
| S1 | 0 | 1 | 0 | 1 | 0 | 0 |
| S2 | 1 | 0 | 0 | 0 | 0 | 1 |
| S3 | 1 | 0 | 0 | 0 | 1 | 0 |
| S4 | 0 | 0 | 1 | 1 | 0 | 0 |
| S5 | 1 | 0 | 0 | 0 | 0 | 1 |

## State Description

S0: North–South traffic moves while East–West traffic stops.

S1: North–South prepares to stop; East–West remains stopped.

S2: East–West traffic moves while North–South traffic stops.

S3: East–West prepares to stop; North–South remains stopped.

S4: Emergency priority is given to North–South.

S5: Emergency priority is given to East–West.

## Design Notes

Only one direction receives a Green signal at any time.

Red and Green are never ON simultaneously for the same direction.

Emergency states maintain Green until the emergency input is cleared.

The output table is implemented using the FSM output logic (case statement) in Verilog.