# FPGA-Based Traffic Light Controller with Emergency Vehicle Priority System Using Verilog HDL

## 1. Project Title

FPGA-Based Traffic Light Controller with Emergency Vehicle Priority System Using Verilog HDL

## 2. Problem Statement

Traffic intersections must efficiently manage vehicle movement while ensuring safety. During emergencies such as ambulances, fire trucks, or police vehicles, the traffic controller should immediately give priority to the emergency route. This project designs a Finite State Machine (FSM)-based traffic light controller in Verilog HDL that automatically cycles through traffic signals and overrides the normal sequence whenever an emergency vehicle is detected.

## 3. Objective

Control traffic signals for a two-road intersection.

Use a Finite State Machine (FSM) for traffic sequencing.

Provide emergency priority for North–South and East–West directions.

Automatically return to normal operation after the emergency clears.

Support simulation using EDA Playground, ModelSim, or Vivado.

Be suitable for FPGA implementation.

## 4. Functional Requirements

Normal Operation

Control traffic lights for two intersecting roads.

Allow only one road to have a Green signal at a time.

Follow the sequence: NS Green → NS Yellow → EW Green → EW Yellow → Repeat.

Emergency Operation

Interrupt the normal sequence immediately.

Give Green signal to the emergency direction.

Keep the opposite direction Red.

Return safely to normal operation after the emergency ends.

## 5. Inputs

| Signal | Description |
| --- | --- |
| clk | System clock |
| rst | Active-high reset |
| emergency_ns | Emergency detected on North–South road |
| emergency_ew | Emergency detected on East–West road |

## 6. Outputs

| Signal | Description |
| --- | --- |
| ns_red | North–South Red LED |
| ns_yellow | North–South Yellow LED |
| ns_green | North–South Green LED |
| ew_red | East–West Red LED |
| ew_yellow | East–West Yellow LED |
| ew_green | East–West Green LED |

## 7. System Features

Finite State Machine (FSM)

Clock-driven operation

Emergency priority override

Safe traffic sequencing

Modular Verilog design

Testbench verification

FPGA-compatible architecture

## 8. Development Tools

HDL: Verilog HDL
Simulation: EDA Playground, ModelSim, Vivado Simulator
Target FPGA (optional): Xilinx Spartan or Artix series

## 9. Project Modules

Traffic FSM: State transitions, output generation, emergency handling.

Timer: Controls green/yellow durations.

Top Module: Connects all modules together.

Testbench: Verifies reset, normal operation, emergency handling, and recovery.

## 10. Expected Results

Correct traffic light sequencing.

Immediate emergency response.

Correct LED outputs.

Successful simulation with waveforms.

FPGA-ready RTL design.

## 11. Applications

Smart traffic intersections

Emergency vehicle management

Digital design education

FPGA learning projects

## 12. Future Enhancements

Vehicle density sensors

Pedestrian crossing support

Countdown timer

IoT monitoring

AI-based traffic optimization