# State Encoding Table

## Purpose

State encoding assigns a unique binary value to every FSM state. The encoded values are stored in the state register and used by the Verilog controller to determine the current operating mode.

## Why 3 Bits?

The controller requires six unique states (S0–S5). Two bits can represent only four states (2² = 4), which is insufficient. Three bits can represent eight states (2³ = 8), so a 3-bit state register is used.

## State Encoding Table

| State | Binary Code | Description |
| --- | --- | --- |
| S0 | 000 | North–South Green, East–West Red |
| S1 | 001 | North–South Yellow, East–West Red |
| S2 | 010 | North–South Red, East–West Green |
| S3 | 011 | North–South Red, East–West Yellow |
| S4 | 100 | Emergency: North–South Green |
| S5 | 101 | Emergency: East–West Green |
| Unused | 110 | Reserved (return to S0) |
| Unused | 111 | Reserved (return to S0) |

## Unused States

Codes 110 and 111 are intentionally left unused. If the FSM ever enters one of these values because of a fault or reset glitch, the default transition should force the controller back to S0.

## Example Verilog Parameters

parameter S0 = 3'b000;
parameter S1 = 3'b001;
parameter S2 = 3'b010;
parameter S3 = 3'b011;
parameter S4 = 3'b100;
parameter S5 = 3'b101;

## Design Notes

Each state has a unique binary value.

Three bits are sufficient for the six required states.

Unused states improve robustness when handled by a default case.

These encodings will be referenced throughout traffic_fsm.v.