# State Transition Table

## Purpose

The state transition table specifies how the Finite State Machine (FSM) moves from one state to the next based on the timer and emergency inputs. This table is the blueprint for the next-state logic in the Verilog design.

## Transition Conditions

timer_done = Green or Yellow timer has expired.

emergency_ns = Emergency vehicle detected on the North–South road.

emergency_ew = Emergency vehicle detected on the East–West road.

## State Transition Table

| Current State | Condition | Next State |
| --- | --- | --- |
| S0 | emergency_ns = 1 | S4 |
| S0 | emergency_ew = 1 | S5 |
| S0 | timer_done | S1 |
| S1 | emergency_ns = 1 | S4 |
| S1 | emergency_ew = 1 | S5 |
| S1 | timer_done | S2 |
| S2 | emergency_ns = 1 | S4 |
| S2 | emergency_ew = 1 | S5 |
| S2 | timer_done | S3 |
| S3 | emergency_ns = 1 | S4 |
| S3 | emergency_ew = 1 | S5 |
| S3 | timer_done | S0 |
| S4 | emergency_ns = 1 | S4 |
| S4 | emergency_ns = 0 | S0 |
| S5 | emergency_ew = 1 | S5 |
| S5 | emergency_ew = 0 | S0 |
| 110 or 111 | Any condition | S0 (safe recovery) |

## Transition Summary

After reset, the FSM starts in S0.

The timer controls normal state progression: S0 → S1 → S2 → S3 → S0.

Emergency requests have higher priority than timer events.

The FSM remains in an emergency state until the corresponding emergency signal becomes LOW.

Unused states automatically recover to S0.

## How This Maps to Verilog

The transition table will be implemented using a combinational `case(current_state)` block that calculates `next_state`. The state register will then update to `next_state` on the rising edge of the clock.